- Technical Specifications (UPS for Desktop Workstation)

General Technical Requirements

The manufacturing vendor(s) should be in existence for at least five (5) years and offering the same brand for an equivalent period of time. The offered brand should be amongst the top five (5) in the local market.

Licensing for all software (where applicable) must be registered through Regional Headquarters, under which Nepal falls, of the global software vendors for ensuring convenient upgradeability and renewal (as and when necessary)

Bidders should provide authorization letter from respective software vendors for third party software.

Electrical Power: All active (powered) equipment must operate on voltage range:  220v +/- 20v, and frequency range: 50Hz +/- 2Hz. All active equipment must include power plugs standard in Nepal.

Environmental: Unless otherwise specified, all equipment must operate in environments of 4-45 degrees centigrade, 20-85 percent relative humidity, and 0-40 grams per cubic meter of dust.

Safety: Unless otherwise specified, all equipment must operate at noise levels no greater than 55 decibels.

All electronic equipment that emits electromagnetic energy must be certified as meeting US FCC class B or EN 55022 and EN 50082-1, or equivalent emission standards.

| SL No. | Description | Minimum Requirement |
| --- | --- | --- |
|  | Standards Compliance | IEEE standards |
|  | Brand | Internationally Reputed Brand |
|  | Model | To be mentioned |
|  | Output | Output Power Capacity: 390 Watts / 650 VA Nominal Output Voltage: 220V Output Frequency: 47 - 63 Hz Output Connections: Total 4 (3 Universal battery backed with Nepal 3-pin Flat, 1 Filtered only (Printer) outlet.) |
|  | Input | Nominal Input Voltage: 160-260V Input Frequency: 50/60 Hz +/- 3 Hz (auto sensing) Input Connections: Nepal 3-pin flat |
|  | Batteries & Run Time | Battery Type: Maintenance-free and leak proof Typical recharge time: 5 hour(s) Replacement battery: Available Minimum Backup Time at Half Load: 20 Minutes |
|  | Management | DB-9 RS-232, USB Control panel with battery status |
|  | Surge Protection | Full time filtering Meet UL 1449 |
|  | Others | Necessary accessories, cables, Driver & Software CD, Connectors (if any) |
|  | Warranty | 1 (One) Year Full (Replacement), 2 Years Service Warranty |

Technical Support:

Warranty Service.

All UPSs & software should have a warranty period that is in conformance with the specifications supplied in the Technical Specification. During the warranty period, the Supplier will be responsible for the repair/replacement/support of all malfunctioning equipment/software supplied under this tender. The Purchaser will require on-site support for warranty period as specified.

The Tenderer must submit as part of their proposal a detailed plan including required manpower to support the supplied Hardware and Software. The required manpower provided must ensure compliance with the requirements. Manpower can be changed with the advance approval of the Purchaser.

User Support:

The supplier should maintain the following user support

- The Tenderer must have service capability up to the district level.

- A suitable Service Center must be established to ensure a high level of service.

- Service records must be maintained at the Service Center for both hardware and software warranty support. In addition, calls to the Service Center must be tracked including the disposition and subsequent resolution of problem.

- The Tenderer is responsible for notification and installation of new versions/releases of software within mutually agreed timeframes.

- Warranty period for the UPSs shall be 1 years.

Delivery

| Quantity | 195 units |  |
| --- | --- | --- |
| Delivery Schedule |  |  |
| Quantity | Location | Maximum Delivery Time |
| 15 units | Kathmandu | 4 weeks |
| 180 | Kathmandu | 6 weeks |