### Technical Specifications (Matching Servers)

General Technical Requirements

The manufacturing vendor(s) should be in existence for at least five (5) years and offering the similar goods and services for a minimum period of two (2) years. The offered brands, where applicable, should be amongst the top ten (10), according to the assessment of a reputed global market analyst, such as ‘Gartner’, ‘IDC’, etc.

Licensing for all software (where applicable) must be registered through Regional Headquarters, under which Nepal falls, of the global software vendors for ensuring convenient upgradeability and renewal (as and when necessary)

Tenderers’ should provide authorization letter from respective hardware and software manufacturers/ authorized or licensed Resellers/vendors.

Electrical Power: All active (powered) equipment must operate on voltage range:  220v +/- 20v, and frequency range: 50Hz +/- 2Hz. All active equipment must include power plugs standard in Nepal.

Environmental: Unless otherwise specified, all equipment must operate in environments of 4-45°C, 20-85 percent relative humidity, and 0-40 grams per cubic meter of dust.

Safety: Unless otherwise specified, all equipment must operate at noise levels no greater than 55 decibels.

All electronic equipment that emits electromagnetic energy must be certified as meeting US FCC class B or EN 55022 and EN 50082-1, or equivalent emission standards.

| SL No. | Description | Minimum Requirement |
| --- | --- | --- |
|  | Form factor | 1 U Rack mount 19 Inch standard |
|  | Processor(s) | 2-way with 2 x 4-Core or 2 x 6-Core 64-bit processors installed. Intel X5550 or AMD Opteron 2435 processors or better |
|  | Memory | 16GB expandable to 32GB (Intel 1066MHz, AMD 667MHz) |
|  | I/O Slots | 1 PCI Express |
|  | Drive Controller(s) | Dual channel SAS and SATA |
|  | RAID Controller | RAID 0,1 |
|  | Drive bays | 2 |
|  | Hard Disk Drives | 2 SAS or SATA 300GB |
|  | Optical Drives | Internal CD/DVD |
|  | Video | 16MB XGA |
|  | Network Interface | 4 Gigabit Ethernet with TOE iSCSI (minimum 2 ports with TOE), RJ-45 |
|  | Power Supply | 1+1 redundant |
|  | Operating System | None |
|  | Mounting | Sliding rails for 4-post square hole rack |
|  | Warranty | 3 (Three) Years Full Warranty on Hardware |
|  | Others | Necessary accessories, cables, drivers, connectors (if any), English manual |

Service Specification

Commissioning: The equipment shall be installed in appointed rack with operating systems installed and configured. Connectivity to network shall be confirmed and tested.

Training and Training Materials:

Users: Technical support staff.

Training or Instruction Manuals must be provided by the supplier on installation, operation of the equipment and basic troubleshooting and maintenance.

Technical Support and Warranty Service.

All Hardware & Software should have a warranty period that is in conformance with the specifications supplied in the Technical Specification. During the warranty period, the Supplier will be responsible for the repair/replacement/support of all malfunctioning equipment/software supplied under this tender.

User Support:

The supplier should maintain the following user support:

- The Tenderer must have on-site service capability in Kathmandu as per specified technical support and warranty service.

- A suitable Service Center must be established to ensure a high level of service.

- Service records must be maintained at the Service Center for both hardware and software warranty support.

- The Tenderer is responsible for notification and installation of new versions/releases of software within mutually agreed timeframes.

- Warranty period shall be 3 years.

Delivery

| Quantity | 4 sets |  |
| --- | --- | --- |
| Delivery Schedule |  |  |
| Quantity | Location | Maximum Delivery Time |
| 4 Sets | Kathmandu | 6 weeks |