- Technical Specifications (Portable Generators)

General Technical Requirements

The manufacturing vendor(s) should be in existence for at least five (5) years and offering the same brand for an equivalent period of time. The offered brand should be amongst the top five (5) in the local market.

Tenderer should provide authorization letter from respective manufacturer.

Environmental: Unless otherwise specified, all equipment must operate in environments of 4-45 degrees centigrade, 20-85 percent relative humidity, and 0-40 grams per cubic meter of dust.

Safety: Unless otherwise specified, all equipment must operate at noise levels allowed under regulations for such equipments.

All electronic equipment that emits electromagnetic energy must be certified as meeting US FCC class B or EN 55022 and EN 50082-1, or equivalent emission standards.

| SL No. | Description | Minimum Requirement |
| --- | --- | --- |
| 1 | Standards Compliance | Relevant recognized standards to be quoted |
| 3 | Model | To be mentioned for both engine and the generators |
| 4 | Output (generators) | Output Power Capacity: minimum 650W max. 1000 W Nominal Output Voltage: 220V Output Frequency: 50 - 60 Hz Output Connections: Total 4 with Nepal standard 3-pin |
| 5 | Fuel for engine | Diesel or Petrol (4-stroke) |
|  | Engine RPM | To be mentioned |
| 6 | Starting | Pull string or handle torque |
| 7 | Lubricating Oil | To be mentioned |
| 8 | Engine-generator coupling | Preferably single shaft. Pulley-belt driven will not be accepted. |
|  | Mounting | The unit (engine and generator) must be mounted on the same frame, with fixed or detachable lifting and carrying handle. |
| 9 | Others | Necessary accessories, cables, control panel, etc. to include. There should be meter to indicate output voltage. |
|  | Fuel Tank Capacity | Capacity should be such that the unit can run at full load for minimum 4-hours. |
| 10 | Warranty (generator and engine unit) | 1 (One) Years Full (24-hours Replacement & Service) Warranty |
| 11 | Standard spare | - One spare spark-plug (for petrol driven engines) or one injection nozzle (for diesel driven engines) - One set of piston rings - One set of standard tools for both engine and generator for regular servicing and plug or nozzle changing |

Technical Support:

Warranty Service.

During the warranty period, the Supplier will be responsible for the repair/replacement/support of all malfunctioning equipment supplied under this tender. The Purchaser will require on-site support for warranty period as specified.

User Support:

The supplier should maintain the following user support:

- The Tenderer must have service capability up to the district level.

- A suitable Service Center must be established to ensure a high level of service.

- Warranty period for the generators shall be 1 years.

Delivery

| Quantity | 600 units |  |
| --- | --- | --- |
| Delivery Schedule |  |  |
| Quantity | Location | Maximum Delivery Time |
| 20 units | Kathmandu | 4 weeks |
| 580 units | Kathmandu | 6 weeks |